SHOW YOUR FACE!

-Technical rider for stage, sound and design-

STAGE:

Platform 1m x 2m x 1m………………………30x

Platform 1m x 2m x 0.80m………………………. 8x

Table cca. 0.60m x 1.50m x 0.80m………………. 2x

Black dance floor covering platforms 10 x 6m

Black curtain at the back of the stage

8 black “legs” for the sides

SOUND:

Microphones/lines:

Actors:- 7 headset wireless microphones (SEHNHEISER, AKG or similar quality mics)

Musicians: 1x Shure SM 58 Beta or similar

1x Shure SM 57 Beta or similar

2x Shure SM 58 or similar

2x Shure SM 57 or similar

3x DI box or similar

2x Condenser microphones Shure SM 94 or AKG C1000 or similar

Outboard:

-32 channel Mixing board/ 4 groups at least / 5 Aux channels at least (3 prefader/2

postfader)  - Soundcraft Spirit live 4, MIDAS, or similar

-8 channels Compresor/Gate – Drawmer, Focusrite,DBX or similar (please NO

Behringer stuff)

-2 channels 32 EQ

-3 effect processors – 1 Lexicon MPX 550, 1 Yamaha SPX2000 (both are a must), TC D-Two or similar delay

-1 effect processor – Alesis AIRFX (I can bring mine own)

FOH:

-1x 3-way system

-2x Full range speakers

Monitors:

-4x monitors with 3 different lines

- upright pianino

Please Contact us if there are any  problems, some of the stuff we can bring with ourselves. Please notice that any equipment from Behringer or similar quality is NOT an option.

LIGHTS:

Dimmers…………………………………………..36 channels

Lighting console……………………………………1x

PC 1kW with barndoors…………………………...20x

Profile spotlight with shutters, 1kW, 15°-36°……..12x

Follow spot 1 kW…………………………………..1x

PAR 64, CP 62……………………………………..7x

PAR 64, CP 60……………………………………..4x

Floor stands………………………………………...3x

Stands 3m…………………………………………..8x (or 4 hanging points on each side of the stage)

The filters are provided by Betontanc.